Notes on Rtwo Design

Bumper

The bumper relies on live springs. Their collective stiffness constant has been tuned on the order of 200 N/m, so that 1N will depress the spring approximately 5mm. Spring orientation angle, spring thickness, serpentine geometry, and spring height were varied to achieve the correct stiffness.

Injection Molding

Several design changes were made to achieve manufacturability via injection molding. First, manufacturing the original design would have required undercutting. This problem was solved by decomposing the spring into a two-part assembly. Second, draft was added. Finally, injection molding required geometry of similar thickness to avoid warping. In order to achieve this, unnecessary material was removed, leaving a relatively empty shell with maximum wall thickness 5mm.

Speaker Cavity

Broadcasting audio was one of rtwo’s design specifications. Speakers work by creating a pressure gradient between the speaker front and back. Therefore, a speaker requires a wall of infinite dimensions along its plane to achieve maximum effectiveness. However, holes in the top piece are required for wiring. The hole sizes will be minimized.