# 2D Truss Solution via Matlab

(Draft 2 April 24, 2006)

## Introduction

A two bay symmetrical truss with cross diagonals in each bay is loaded at the center bottom node with a vwertical force.  IT is pinned at the left bottom node and supported by a horizontal roller (no vertical displacement) at the lower right node.  Determine the deflections, reactions, and member forces if it is assumed (unrealistically) that all the members have the same area (A) and elastic modulus (E).  Each bay has a width and height of 10 ft.

## Validation problem

>> Modular_2D_Truss(1)

Read 6 nodes.

Node,  BC_Flag, Coordinates

1 11   0  0  0

2 0   0  10  0

3 0   10  0  0

4 0   10  10  0

5 0   20  10  0

6 1   20  0  0

Read 11 elements.

Elem, Type, Connectivity_List

1 1 1 3

2 1 3 6

3 1 1 2

4 1 2 3

5 1 1 4

6 1 3 4

7 1 4 6

8 1 3 5

9 1 5 6

10 1 2 4

11 1 4 5

Maximum element type number = 1

Read 3 essential boundary condition sets.

Node  DOF  Value_EBC

1  1  0

1  2  0

6  2  0

Read 1 point sources.

Node DOF Source_value

3  2  -10

Application properties are:

Elastity modulus     = 30000

Cross-sectional area = 1

Line Load = [ 0 0 ]

Node, DOF, Resultant input sources

3 2 -10

2 DOF Totals =      0   -10

Computed nodal displacements at 6 nodes

Node DOF_1   DOF_2   DOF_3  DOF_4  DOF_5  DOF_6

1 0 0

2 0.00166667 -0.00091153

3 0.000755136 -0.00440126

4 0.000755136 -0.00289098

5 -0.000156394 -0.00091153

6 0.00151027 0

Recovered 3 Reactions

Node, DOF, Value of reaction

1 1 1.77636e-15

1 2 5

6 2 5

2 DOF Totals =     0.0000   10.0000

>> mesh_shrink_plot

>> bc_flags_plot

>> quiver_resultant_load_mesh(0.25)

Using a scale of 0.25 and vector increment of 1

>> deformed_mesh_plot(250)

Suggested scale = 227.208

>> quiver_disp_vec_mesh(0.5)

Using a scale of 0.5 and vector increment of 1

>> quiver_reaction_vec_mesh(0.25)

Using a scale of 0.25 and vector increment of 1

>> truss_el_force_value

## Modular source on Matlab script

The planar truss script is listed first.  It is followed by all the current supporting functions for general FEA, listed in alphebetical order.

function Modular_2D_Truss (load_pt)

%.............................................................

%   Classic planar truss for point loads (& line load soon)

%             XY COORDINATES CLOSED FORM INTEGRALS

%.............................................................

% pre_e = # of dummy items before el_type & connectivity

% pre_p = # of dummy items before BC_flag % coordinates

pre_e = 0 ; pre_p = 0 ;       % default, consistent with plots

if ( nargin == 0 )                 ; % check for optional data

load_pt = 0                         ; % no point source data

end % if from argument count

%         Application and element dependent controls

n_g = 2  ; % number of DOF per node (axial_d, transverse_d)

n_q = 0  ; % number of quadrature points required

n_r = 1  ; % number of rows in B_e matrix

%              Read mesh input data files

[n_m, n_s, P, x, y, z]          = get_mesh_nodes    (pre_p) ;

[n_e, n_n, n_t, el_type, nodes] = get_mesh_elements (pre_e) ;

n_d = n_g*n_m                ; % system degrees of freedom (DOF)

n_i = n_g*n_n                ; % number of DOF per element

S = zeros (n_d, n_d) ; C = zeros (n_d, 1)      ; % initalize sums

M = zeros (n_d, n_d) ;                         ; % initalize sums

%      Extract EBC flags from packed integer flag P

[EBC_flag] = get_ebc_flags (n_g, n_m, P)      ; % unpack flags

EBC_count  = sum( sum ( EBC_flag > 0 ) )      ; % # of EBC

%                  Read EBC values, if any

if ( EBC_count > 0 )                            ; % need EBC data

[EBC_value] = get_ebc_values (n_g, n_m, EBC_flag) ; % read data

end % if any EBC data expected

%    Read point loads or moments, if any, and insert in C

if ( load_pt > 0 )                       ; % need point loads data

[C] = get_and_add_point_sources (n_g, n_m, C); % add point loads

end % if any point source expected

%  ============== ASSUMING HOMOGENOUS PROPERTIES =================

%    GENERATE ELEMENT MATRICES AND ASSYMBLE INTO SYSTEM

%  Assemble n_d by n_d square matrix terms from n_e by n_e

for j = 1:n_e ; % loop over elements ====>> ====>> ====>> ====>>

S_e = zeros (n_i, n_i) ; M_e = zeros (n_i, n_i) ; % sys arrays

C_p = zeros (n_i, 1)   ; C_e = zeros (n_i, 1)   ; % sys arrays

s_L = zeros (n_i, n_i) ; m_L = zeros (n_i, n_i) ; % loc arrays

t_L = zeros (n_i, n_i) ; c_L = zeros (n_i, 1)   ; % loc arrays

e_nodes = nodes (j, 1:n_n)                      ; % connectivity

%               SET ELEMENT PROPERTIES & GEOMETRY

Option = 1 ; % select analysis case

[A, E, Line_e, Rho] = set_2D_truss_properties (n_n, Option) ;

%-->   find member length and direction cosines

dx  = x(e_nodes(2)) - x(e_nodes(1)) ; % x length

dy  = y(e_nodes(2)) - y(e_nodes(1)) ; % y length

L_e = sqrt (dx * dx + dy * dy)      ; % total length

cx  = dx / L_e ; cy   = dy / L_e    ; % direction cosines

%      ELEMENT CONDUCTION AND INTERNAL SOURCE MATRICES

% for q = 1:n_q ; % Loop over quadrature points ----> ---->

%   Linear axial bar and cubic bending. DOF = u, v, u, v

%   Form arrays in local axes, transform.     1  2  3  4

%                 stiffness

s_L = [ 1,    0,   -1,    0 ;

0,    0,    0,    0 ;

-1,    0,    1,    0 ;

0,    0,    0,    0 ] * E * A / L_e ;

%  Map line load to node forces ONLY

if ( any (Line_e) )          ; % then form forcing vector

fprintf ('WARNING: line loads not yet active. \n')

end % if for set up line load nodal resultants

%        Optional local mass matrix

if ( Rho > 0 )

m_L = [140, 0,       70,  0 ;

0,   0,       0,   0 ;

70,  0,       140, 0 ;

0,   0,       0,   0 ] * Rho * A * L_e ;

end % if mass requested

% end % for loop over n_q element quadrature points    <---- <----

%     Define local to system DOF transformation matrix

t_L = [ cx   cy   0   0  ; % (inverse t_L = transpose t_L)

-cy   cx   0   0  ;

0    0    cx  cy ; % joint 2

0    0   -cy  cx ] ;

%         Transform from local to system

S_e = t_L' * s_L * t_L ; M_e = t_L' * m_L * t_L ;

C_e = t_L' * c_L ;

%         SCATTER TO (ASSEMBLE INTO) SYSTEM ARRAYS

% Insert completed element matrices into system matrices

[rows] = get_element_index (n_g, n_n, e_nodes); % eq numbers

S (rows, rows) = S (rows, rows) + S_e  ; % add to system sq

C (rows) = C (rows) + C_e              ; % add to sys column

end % for each j element in mesh   <<====  <<====  <<====   <<====

%    ALLOCATE STORAGE FOR OPTIONAL REACTION RECOVERY

if ( EBC_count > 0 )                        ; % reactions occur

[EBC_row, EBC_col] = save_reaction_matrices (EBC_flag, S, C);

end % if essential BC exist (almost always true)

%                   ECHO PROPERTIES (add Rho)

fprintf ('Application properties are: \n')

fprintf ('Elastity modulus     = %g \n', E)

fprintf ('Cross-sectional area = %g \n', A)

fprintf ('Line Load = [ %g %g ] \n', Line_e(1), Line_e(2))

%         ENFORCE ESSENTIAL BOUNDARY CONDITIONS

save_resultant_load_vectors (n_g, C)

if ( all ( C == 0 ) ) % then null solution

fprintf ('WARNING: No loads or support movement applied. \n')

end % if null solution

[S, C] = enforce_essential_BC (EBC_flag, EBC_value, S, C);

%             COMPUTE SOLUTION & SAVE

T = S \ C                 ; % Compute displacements & rotations

list_save_displacements_results (n_g, n_m, T)

%          OPTIONAL REACTION RECOVERY & SAVE

if ( EBC_count > 0 )                    ; % reactions exist ?

[EBC_react] = recover_reactions_print_save (n_g, n_d, ...

EBC_flag, EBC_row, EBC_col, T); % reaction to EBC

end % if EBC exist

%       POST-PROCESS ELEMENT REACTIONS (MEMBER FORCES)

% output_2D_truss_el_reactions (n_e, n_g, n_n, n_q, nodes, x, y, T)

% End finite element calculations.

% See /home/mech517/public_html/Matlab_Plots for graphic options

% http://www.owlnet.rice.edu/~mech517/help_plot.html for help

% end of Modular_2D_Truss

%  +++++++++++++ functions in alphabetical order +++++++++++++++++

function [S, C] = enforce_essential_BC (EBC_flag, EBC_value, S, C)

% modify system linear eqs for essential boundary conditions

% (by trick to avoid matrix partitions, loses reaction data)

n_d = size (C, 1)                        ; % number of DOF eqs

if ( size (EBC_flag, 2) > 1 ) ; % change to vector copy

flag_EBC  = reshape ( EBC_flag',  1, n_d) ;

value_EBC = reshape ( EBC_value', 1, n_d) ;

else

flag_EBC  = EBC_flag  ;

value_EBC = EBC_value ;

end % if

for j = 1:n_d                % check all DOF for essential BC

if ( flag_EBC (j) )                        % then EBC here

%    Carry known columns*EBC to RHS. Zero that column and row.

%    Insert EBC identity, 1*EBC_dof = EBC_value.

EBC = value_EBC (j)             ; % recover EBC value

C (:) = C (:) - EBC * S (:, j) ; % carry known column to RHS

S (:, j) = 0 ; S (j, :) = 0    ; % clear, restore symmetry

S (j, j) = 1 ; C (j) = EBC     ; % insert identity into row

end % if EBC for this DOF

end % for over all j-th DOF

% end enforce_essential_BC (EBC_flag, EBC_value, S, C)

function [a, b, c, center, two_A] = form_T3_geom_constants (x, y, e_nodes)

% Planar 3 node triangle geometry: H_i (x,y) = (a_i + b_i*x + c_i*y)/two_a

%                   define nodal coordinates, ccw: i, j, k

x_e = x(e_nodes) ; y_e = y(e_nodes)        ; % coord at el nodes

x_i = x_e(1) ; x_j = x_e(2) ; x_k = x_e(3) ; % change notation

y_i = y_e(1) ; y_j = y_e(2) ; y_k = y_e(3) ; % change notation

%  define centroid coordinates (quadrature point)

center (1) = (x_i + x_j + x_k)/3 ;

center (2) = (y_i + y_j + y_k)/3 ;

%  geometric parameters: H_i (x,y) = (a_i + b_i*x + c_i*y)/two_a

a_i = x_j * y_k - x_k * y_j ; b_i = y_j - y_k ; c_i = x_k - x_j ;

a_j = x_k * y_i - x_i * y_k ; b_j = y_k - y_i ; c_j = x_i - x_k ;

a_k = x_i * y_j - x_j * y_i ; b_k = y_i - y_j ; c_k = x_j - x_i ;

a (1:3) = [a_i, a_j, a_k] ;

b (1:3) = [b_i, b_j, b_k] ;

c (1:3) = [c_i, c_j, c_k] ;

%  calculate twice element area

two_A = a_i + a_j + a_k            ; % = b_j*c_k - b_k*c_j also

% end form_T3_geom_constants (x, y, e_nodes)

function [C] = get_and_add_point_sources (n_g, n_m, C)

load msh_load_pt.tmp            ; % node, DOF, value (eq. number)

n_u = size(msh_load_pt, 1)      ; % number of point sources

if ( n_u < 1 )                  ; % missing data

error ('No load_pt data in msh_load_pt.tmp')

end % if user error

fprintf ('Read %g point sources. \n', n_u)

fprintf ('Node DOF Source_value \n')

for j = 1:n_u                    ; % non-zero Neumann pts

node  = msh_load_pt (j, 1)         ; % global node number

DOF   = msh_load_pt (j, 2)         ; % local DOF number

value = msh_load_pt (j, 3)         ; % point source value

fprintf ('%g  %g  %g \n', node, DOF, value)

Eq    = n_g * (node - 1) + DOF     ; % row in system matrix

C (Eq) = C (Eq) + value     ; % add to system column matrix

end % for each EBC

fprintf ('\n')

% end get_and_add_point_sources (n_g, n_m, C)

function [EBC_flag] = get_ebc_flags (n_g, n_m, P)

EBC_flag = zeros(n_m, n_g)           ; % initialize

for k = 1:n_m                        ; % loop over all nodes

if ( P(k) > 0 )                    ; % at least one EBC here

[flags] = unpack_pt_flags (n_g, k, P(k)) ; % unpacking

EBC_flag (k, 1:n_g) = flags (1:n_g)      ; % populate array

end % if EBC at node k

end % for loop over all nodes

% end get_ebc_flags

function [EBC_value] = get_ebc_values (n_g, n_m, EBC_flag)

EBC_count = sum( sum ( EBC_flag > 0 ) ) ; % # of EBC

EBC_value = zeros(n_m, n_g)  ; % initialize to zero

load msh_ebc.tmp            ; % node, DOF, value (eq. number)

n_c = size(msh_ebc, 1)      ; % number of constraints

fprintf ('Read %g essential boundary condition sets. \n', n_c)

fprintf ('Read %g essential boundary condition sets. \n', n_c)

if ( n_c ~= EBC_count ) % then probable user error

fprintf ('WARNING, expected %g EBC sets. \n', EBC_count)

end % if error expected

fprintf ('Node  DOF  Value_EBC \n')

for j = 1:n_c                    ; % loop over ebc inputs

node  = round (msh_ebc (j, 1)) ; % node in mesh

DOF   = round (msh_ebc (j, 2)) ; % DOF # at node

value = msh_ebc (j, 3)         ; % EBC value

% Eq  = n_g * (node - 1) + DOF ; % row in system matrix

EBC_value (node, DOF) = value  ; % insert value in array

fprintf ('%g  %g  %g \n', node, DOF, value)

if ( EBC_flag (node, DOF) == 0 ) % check data consistency

fprintf ('WARNING: EBC but no flag at node %g & DOF %g. \n', ...

node, DOF)

% EBC_flag (node, DOF) = 1; % try to recover from data error

end % if common user error

end % for each EBC

EBC_count = sum (sum ( EBC_flag > 0 )) ; % check input data

if ( EBC_count ~= n_c )                ; % probable user error

fprintf ('WARNING: mismatch in bc_flag count & msh_ebc.tmp \n')

end % if user error

fprintf ('\n')

% end get_ebc_values

function [rows] = get_element_index (n_g, n_n, e_nodes)

% calculate system DOF numbers of element, for gather, scatter

rows = zeros (1, n_g*n_n)           ; % allow for node = 0

for k = 1:n_n                       ; % loop over element nodes

global_node = round (e_nodes (k)) ; % corresponding sys node

for i = 1:n_g                     ; % loop over DOF at node

eq_global  = i + n_g * (global_node - 1) ; % sys DOF, if any

eq_element = i + n_g * (k           - 1) ; % el DOF number

if ( eq_global > 0 )                ; % check node=0 trick

rows (1, eq_element) = eq_global  ; % valid DOF > 0

end % if allow for omitted nodes

end % for DOF i                       % end local DOF loop

end % for each element node             % end local node loop

% end get_element_index

function [n_e, n_n, n_t, el_type, nodes] = get_mesh_elements (pre_e) ;

% MODEL input file controls (for various data generators)

if (nargin == 0)  ; % default to no proceeding items in data

pre_e = 0       ; % Dummy items before el_type & connectivity

end % if

load  msh_typ_nodes.tmp        ; % el_type, connectivity list (3)

n_e = size (msh_typ_nodes,1)   ; % number of elements

if ( n_e == 0 )                ; % data file missing

error ('Error missing file msh_typ_nodes.tmp')

end % if error

n_n = size (msh_typ_nodes,2) - pre_e - 1 ; % nodes per element

el_type = round (msh_typ_nodes(:, pre_e+1)); % el type number >= 1

n_t     = max(el_type)                   ; % number of element types

nodes (1:n_e, 1:n_n) = msh_typ_nodes (1:n_e, (pre_e+2:pre_e+1+n_n));

fprintf ('Read %g elements. \n', n_e)

fprintf ('Elem, Type, Connectivity_List \n')

for j = 1:n_e

if ( n_n == 1 )

fprintf ('%g %g %g                \n', j, el_type(j), nodes(j,:));

elseif ( n_n == 2 )

fprintf ('%g %g %g %g             \n', j, el_type(j), nodes(j,:));

elseif ( n_n == 3 )

fprintf ('%g %g %g %g %g          \n', j, el_type(j), nodes(j,:));

elseif ( n_n == 4 )

fprintf ('%g %g %g %g %g %g       \n', j, el_type(j), nodes(j,:));

elseif ( n_n == 5 )

fprintf ('%g %g %g %g %g %g %g    \n', j, el_type(j), nodes(j,:));

elseif ( n_n == 6 )

fprintf ('%g %g %g %g %g %g %g %g \n', j, el_type(j), nodes(j,:));

else

fprintf ('%g %g \n', j, el_type(j)); dips( nodes(j,:));

end % if

end % for each element

fprintf ('Maximum element type number = %g \n \n', n_t)

% end get_mesh_elements

function [n_m, n_s, P, x, y, z] = get_mesh_nodes (pre_p) ;

% MODEL input file controls (for various data generators)

if (nargin == 0) % set usual default

pre_p = 0 ; % Dummy items before BC_flag % coordinates

end % if

%       READ MESH AND EBC_FLAG INPUT DATA

% specific problem data from MODEL data files (sequential)

load msh_bc_xyz.tmp       ; % bc_flag, x-, y-, z-coords

n_m = size (msh_bc_xyz,1) ; % number of nodal points in mesh

if ( n_m == 0 )           ; % data missing !

error ('Error missing file msh_bc_xyz.tmp')

end % if error

n_s = size (msh_bc_xyz,2) - pre_p - 1 ; % number of space dimensions

msh_bc_xyz (:, (pre_p+1))= round (msh_bc_xyz (:, (pre_p+1)));

P = msh_bc_xyz (1:n_m, (pre_p+1)) ; % integer Packed BC flag

x = msh_bc_xyz (1:n_m, (pre_p+2)) ; % extract x column

y (1:n_m, 1) = 0.0 ;  z (1:n_m, 1) = 0.0 ; % default to zero

if (n_s > 1 )                                 ; % check 2D or 3D

y = msh_bc_xyz (1:n_m, (pre_p+3))           ; % extract y column

end % if 2D or 3D

if ( n_s == 3 )                               ; % check 3D

z = msh_bc_xyz (1:n_m, (pre_p+4))           ; % extract z column

end % if 3D

%b if ( pre_p ~= 1 ) % not given node number, sequential data

fprintf ('Read %g nodes. \n', n_m)

fprintf ('Node,  BC_Flag, Coordinates \n')

for j = 1:n_m                                 ; % list nodes

fprintf ('%g %g   %g  %g  %g \n', j, P(j), x(j), y(j), z(j)) ;

end % for j DOF

fprintf ('\n')

% end get_mesh_nodes

function list_save_beam_displacements (n_g, n_m, T)

fprintf ('\n') ;

fprintf('Node  Y_displacement  Z_rotation at %g nodes \n', n_m)

T_matrix = reshape (T, n_g, n_m)'            ; % pretty shape

% save results (displacements) to MODEL file: node_results.tmp

fid = fopen('node_results.tmp', 'w') ; % open for writing

for j = 1:n_m                        ; % node loop, save displ

fprintf (fid, '%g %g \n',  T_matrix (j, 1:n_g)) ; % to file

fprintf ('%g %g %g \n', j, T_matrix (j, 1:n_g)) ; % to screen

end % for j DOF

% end list_save_beam_displacements (n_g, n_m, T)

function list_save_displacements_results (n_g, n_m, T)

fprintf('Computed nodal displacements at %g nodes \n', n_m)

fprintf('Node DOF_1   DOF_2   DOF_3  DOF_4  DOF_5  DOF_6 \n')

T_matrix = reshape (T, n_g, n_m)'            ; % pretty shape

% save results (displacements) to MODEL file: node_results.tmp

fid = fopen('node_results.tmp', 'w')    ; % open for writing

for j = 1:n_m                           ; % save displacements

if ( n_g == 1 )

fprintf (fid, '%g                 \n', T_matrix (j, 1:n_g)) ;

fprintf ('%g %g                \n', j, T_matrix (j, 1:n_g)) ;

elseif ( n_g == 2 )

fprintf (fid, '%g %g              \n', T_matrix (j, 1:n_g)) ;

fprintf ('%g %g %g             \n', j, T_matrix (j, 1:n_g)) ;

elseif ( n_g == 3 )

fprintf (fid, '%g %g %g           \n', T_matrix (j, 1:n_g)) ;

fprintf ('%g %g %g %g          \n', j, T_matrix (j, 1:n_g)) ;

elseif ( n_g == 4 )

fprintf (fid, '%g %g %g %g        \n', T_matrix (j, 1:n_g)) ;

fprintf ('%g %g %g %g %g       \n', j, T_matrix (j, 1:n_g)) ;

elseif ( n_g == 5 )

fprintf (fid, '%g %g %g %g %g     \n', T_matrix (j, 1:n_g)) ;

fprintf ('%g %g %g %g %g %g    \n', j, T_matrix (j, 1:n_g)) ;

elseif ( n_g == 6 )

fprintf (fid, '%g %g %g %g %g %g  \n', T_matrix (j, 1:n_g)) ;

fprintf ('%g %g %g %g %g %g %g \n', j, T_matrix (j, 1:n_g)) ;

else

error('reformat list_save_displacements_results for n_g > 6.')

end % if

end % for j DOF

fprintf ('\n') ;

% end list_save_displacements_results (T)

function list_save_temperature_results (T)

n_m = size (T, 1)                          ; % get size

fprintf('Temperature at %g nodes \n', n_m) ; % header

% save results (temperature) to MODEL file: node_results.tmp

fid = fopen('node_results.tmp', 'w')       ; % open for writing

for j = 1:n_m                              ; % save temperature

fprintf ( fid, '%g \n', T (j))           ; % print

fprintf (' %g %g \n', j, T (j))          ; % sequential save

end % for j DOF

% end list_save_temperature_results (T)

function output_PlaneStress_stresses(n_e, n_g, n_n, n_q, nodes, x,y,T)

%       POST-PROCESS ELEMENT STRESS RECOVERY & SAVE

fid = fopen('el_qp_xyz_fluxes.tmp', 'w')    ; % open for writing

fprintf ('\n')                              ; % blank line

fprintf('Elem, QP, X_qp, Y_qp \n')                         ;% header

fprintf('Elem, QP, Stress_qp: xx        yy          xy \n');% header

for j = 1:n_e                      ; % loop over elements   ====>>

e_nodes = nodes (j, 1:n_n)               ; % connectivity

[a, b, c, center, two_A] = form_T3_geom_constants (x, y, e_nodes);

[t_e, Body_e, E_e] = set_constant_plane_stress_prop; % properties

%  get DOF numbers for this element, gather solution

[rows] = get_element_index (n_g, n_n, e_nodes) ; % eq numbers

T_e = T (rows)                          ; % gather element DOF

for q = 1:n_q        ; % Loop over element quadrature points ---->

%   H_i (x,y) = (a_i + b_i*x + c_i*y)/two_A    % interpolations

B_e (1, 1:2:5) = b (1:3)/two_A ; B_e (2, 2:2:6) = c (1:3)/two_A ;

B_e (3, 1:2:5) = c (1:3)/two_A ; B_e (3, 2:2:6) = b (1:3)/two_A ;

%  COMPUTE GRADIENT & HEAT FLUX, SAVE LOCATION AND VALUES

Strains = B_e * T_e                      ; % mechanical strain

Stress = E_e * Strains                   ; % mechanical stress

fprintf (fid,'%g %g %g %g %g  \n',  center(1), center(2), ...

Stress(1), Stress(2), Stress(3));% save

fprintf ('%g %g %g %g  \n',    j, q, center(1:2));% prt

fprintf ('%g %g %g %g %g  \n', j, q, Stress(1:3));% prt

fprintf ('\n')                                   ;% prt

end % for loop over n_q element quadrature points          <----

end % for each j element in mesh

% end output_PlaneStress_stresses (n_e, n_g, n_n, n_q, nodes, x, y, T)

function output_T3_heat_flux (n_e, n_g, n_n, n_q, nodes, x, y, T)

%       POST-PROCESS ELEMENT HEAT FLUX RECOVERY & SAVE

fid = fopen('el_qp_xyz_fluxes.tmp', 'w')    ; % open for writing

fprintf ('\n')                              ; % blank line

fprintf('Elem, X_qp, Y_qp, HeatFlux_x, HeatFlux_y \n');% header

for j = 1:n_e                      ; % loop over elements   ====>>

e_nodes = nodes (j, 1:n_n)               ; % connectivity

[a, b, c, center, two_A] = form_T3_geom_constants (x, y, e_nodes);

[t_e, Body_e, E_e] = set_constant_plane_stress_prop; % properties

%  get DOF numbers for this element, gather solution

[rows] = get_element_index (n_g, n_n, e_nodes) ; % eq numbers

T_e = T (rows)                          ; % gather element DOF

for q = 1:n_q        ; % Loop over element quadrature points ---->

%   H_i (x,y) = (a_i + b_i*x + c_i*y)/two_A    % interpolations

B_e (1, 1:3) = b(1:3) / two_A            ; % dH/dx

B_e (2, 1:3) = c(1:3) / two_A            ; % dH/dy

%  COMPUTE GRADIENT & HEAT FLUX, SAVE LOCATION AND VALUES

Gradient = B_e * T_e                        ; % gradient vector

HeatFlux = E_e * Gradient                   ; % heat flux vector

fprintf (fid, '%g %g %g %g \n',  center(1:2), HeatFlux(1:2));% save

fprintf ('%g %g %g %g %g \n', j, center(1:2), HeatFlux(1:2));% prt

end % for loop over n_q element quadrature points          <----

end % for each j element in mesh                            <<====

% end output_T3_heat_flux (n_e, n_g, n_n, n_q, nodes, x, y, T)

function [EBC_react] = recover_reactions_print_save (n_g, n_d, ...

EBC_flag, EBC_row, EBC_col, T)

% get EBC reaction values by using rows of S & C (before EBC)

n_d = size (T, 1)                     ; % number of system DOF

% n_c x 1 = n_c x n_d  * n_d x 1 +  n_c x 1

EBC_react = EBC_row * T - EBC_col     ; % matrix reactions (+-)

% save reactions (forces) to MODEL file: node_reaction.tmp

fprintf ('Recovered %g Reactions \n', ...

sum (sum (EBC_flag > 0)))   ; % header

fprintf ('Node, DOF, Value of reaction \n')

fid = fopen('node_reaction.tmp', 'w') ; % open for writing

if ( size (EBC_flag, 2) > 1 )            ; % change to vector copy

flag_EBC  = reshape ( EBC_flag',  1, n_d) ; % changed

else

flag_EBC  = EBC_flag                      ; % original vector

end % if

Totals = zeros (1, n_g)               ; % zero input totals

kount = 0                             ; % initialize counter

for j = 1:n_d                         ; % extract all EBC reactions

if ( flag_EBC(j) )                      ; % then EBC here

%   Output node_number, component_number, value, equation_number

kount = kount + 1                     ; % copy counter

node  = ceil(j/n_g)                   ; % node at DOF j

j_g   = j - (node - 1)*n_g            ; % 1 <= j_g <= n_g

React = EBC_react (kount, 1)          ; % reaction value

fprintf ( fid, '%g %g %g \n', node, j_g, React);% save

fprintf ('%g %g %g \n', node, j_g, React);     % print

Totals (j_g) = Totals (j_g) + React   ; % sum all components

end % if EBC for this DOF

end % for over all j-th DOF

fprintf ('%g DOF Totals = ', n_g) ; disp(Totals)     ; % echo totals

fprintf ('\n')                                   ; % Skip a line

% end recover_reactions_print_save (EBC_row, EBC_col, T)

function [EBC_row, EBC_col] = save_reaction_matrices (EBC_flag, S, C)

n_d       = size (C, 1)                  ; % number of system DOF

EBC_count = sum (sum (EBC_flag))         ; % count EBC & reactions

EBC_row   = zeros(EBC_count, n_d)        ; % reaction data

EBC_col   = zeros(EBC_count, 1)          ; % reaction data

if ( size (EBC_flag, 2) > 1 )            ; % change to vector copy

flag_EBC  = reshape ( EBC_flag',  1, n_d) ; % changed

else

flag_EBC  = EBC_flag                      ; % original vector

end % if

kount = 0                                ; % initialize counter

for j = 1:n_d      % System DOF loop, check for displacement BC

if ( flag_EBC (j) )                    ; % then EBC here

%   Save reaction data to be destroyed by EBC solver trick

kount = kount + 1                    ; % copy counter

EBC_row(kount, 1:n_d) = S (j, 1:n_d) ; % copy reaction data

EBC_col(kount, 1)     = C (j)        ; % copy reaction data

end % if EBC for this DOF

end % for over all j-th DOF                % end sys DOF loop

% end save_reaction_matrices (S, C, EBC_flag)

function save_resultant_load_vectors (n_g, C)

% save resultant forces to MODEL file: node_resultants.tmp

n_d = size (C, 1)                     ; % number of system DOF

fprintf ('\n')                        ; % Skip a line

% fprintf ('Node, DOF, Resultant Force (1) or Moment (2)  \n')

fprintf ('Node, DOF, Resultant input sources \n')

fid = fopen('node_resultant.tmp', 'w'); % open for writing

Totals = zeros (1, n_g)               ; % zero input totals

for j = 1:n_d                         ; % extract resultants

if ( C (j) ~= 0. )                      ; % then source here

% Output node_number, component_number, value

node  = ceil(j/n_g)                   ; % node at DOF j

j_g   = j - (node - 1)*n_g            ; % 1 <= j_g <= n_g

value = C (j)                         ; % resultant value

fprintf ( fid, '%g %g %g %g \n', node,j_g,value,j);% save

fprintf ('%g %g %g \n', node, j_g, value);     % print

Totals (j_g) = Totals (j_g) + value   ; % sum all inputs

end % if non-zero for this DOF

end % for over all j-th DOF

fprintf ('%g DOF Totals = ', n_g) ; disp(Totals)      ; % echo totals

% end save_resultant_load_vectors (n_g, n_m, C)

function [I, E, Rho, Line_e]=set_constant_beam_prop (n_n, Option);

if ( nargin == 1 )

Option = 1 ;

elseif ( nargin == 0 )

n_n = 2 ; Option = 1 ;

end % if problem Option number

Line_e = zeros (n_n, 1)         ; % default line load at nodes

switch  Option

case 1 % Propped cantilever with uniform load, L, L/4

% Wall reactions: V=37*Line*L/64 M=7*Line*L^2/64

% Roller reaction: R= 43*Line*L/64

% Total vertical load: 5*Line*L/4

%       *-----(1)-----*-----(2)-----*--(3)--*     EI

% Fixed@1     L/2     2      Roller@3  L/4  4

I = 1.0 ; E = 1.0 ; Rho = 0.0 ; Line_e = [1.0; 1.0] ;

case 2 % cantilever with uniform load, L

I = 1.0 ; E = 1.0 ; Rho = 0.0 ; Line_e = [1.0; 1.0] ;

otherwise

I = 1.0 ; E = 1.0 ; Rho = 0.0; % default shape & material

end % switch

% end set_constant_beam_prop;

function [t_e, Body_e, E_e] = set_constant_plane_stress_prop ;

t_e = 1 ; Body_e (1:2) = 0.             ; % defaults

%                         case 1

t_e          = 5e-3                     ; % thickness

Body_e (1:2) = [5e5, 0.]                ; % components

E   = 15e9                              ; % Elastic modulus

nu  = 0.25                              ; % Poisson's ratio

%                    plane stress

E_v = E/(1 - nu^2)                            ; % constant

E_e (1, 1) = E_v      ; E_e (1, 2) = E_v * nu ; % non-zero term

E_e (2, 1) = E_v * nu ; E_e (2, 2) = E_v      ; % non-zero term

E_e (3, 3) = E_v * (1 - nu) / 2               ; % non-zero term

%end set_constant_plane_stress_prop

function [t_e, Q_e, E_e] = set_constant_2D_conduction_prop

% Manually set constant element properties (Fig 11.9 text)

q_e = 0. ; t_e = 1.                  ; % defaults

% case 1

Kx  = 8. ; Ky = 8. ; Kxy = 0.        ; % thermal conductivity

% case 2

kx  = 1. ; Ky = 1. ; Kxy = 0. ;

% insert

E_e = zeros (2, 2)                      ; % constitutive matrix

E_e (1, 1) = Kx   ; E_e (1, 2) = Kxy    ; % non-zero term

E_e (2, 1) = Kxy  ; E_e (2, 2) = Ky     ; % non-zero term

% end set_constant_2D_conduction_prop

function [A, E, I, Line_e, Rho] = set_2D_frame_properties (n_n, Option)

if ( nargin == 1 )

Option = 1 ;

elseif ( nargin == 0 )

n_n = 2 ; Option = 1 ;

end % if problem Option number

Line_e = zeros (n_n, 1)         ; % default line load at nodes

switch Option

case 1

%     Weaver plane frame example  X--------\ F_y=-32 K,

%     E=10000ksi, A=10 in sq     2   (1)  1 \  M_z=-1050 in-K,

%     I=1000 in^4, L_1=100 in             (2)\  at node 1.

%     L_2x=100 in, L_2y=-75 in                \ [no line load]

%     Node 1 disp: -0.02026 in, -0.09936 in,   X

%     and -0.001798 radians                    3

%     Reactions, node 2: 20.261 K, 1.1378 K, 236.65 in K

%     node 3:-20.261 K, 30.862 K, -639.52 in-K

A=10; I=1e3; E=1e4; Rho=0; Line_e = [0.0; 0.0] ;

case 2 % cantilever with uniform load, L

A = 1 ; I = 1 ; E = 1 ; Rho = 0 ; Line_e = [1.0; 1.0] ;

otherwise

A = 1 ; I = 1 ; E = 1 ; Rho = 0; % default shape & material

end % switch

% end set_2D_frame_properties (n_n, Option)

function [A, E, Line_e, Rho] = set_2D_truss_properties (n_n, Option)

if ( nargin == 1 )

Option = 1 ;

elseif ( nargin == 0 )

n_n = 2 ; Option = 1 ;

end % if problem Option number

Line_e = zeros (n_n, 1)         ; % default line load at nodes

switch Option

case 1

%   2       4        5    Meek's Example 7.2 truss

%   *--(10)-*--(11)-*     E = 30,000 ksi,  A = 1 in^2

%   |\(4)  /|\(7)  /|     Two 10 inch bays

%   |  \ /  |  \ /  |     Max vertical deflection @ 3

% (3)   X  (6)  X  (9)    is -4.4013E-03 inches

%   |  / \  |  / \  |

%   |/(5)  \|/(8)  \|     Reactions are 5K each at 1, 6

% 1 #--(1)--*--(2)--o 6

% Pin      3|       Roller

%           v P=10K

A=1; E=30e3 ; Rho=0; Line_e = [0.0; 0.0] ;

otherwise

A = 1 ; E = 1 ; Rho = 0; % default shape & material

end % switch

% end set_2D_truss_properties (n_n, Option)

function [flags] = unpack_pt_flags (n_g, N, flag)

% unpack n_g integer flags from the n_g digit flag at node N

% integer flag contains (left to right) f_1 f_2 ... f_n_g

full = flag                         ; % copy integer

check = 0                           ; % validate input

%b size(flag)

%b size(full)

for Left2Right = 1:n_g              ; % loop over local DOF at k

Right2Left = n_g + 1 - Left2Right ; % reverse direction

work = floor (full / 10)          ; % work item

keep = full - work * 10           ; % work item

flags (Right2Left) = keep         ; % insert into array

full = work                       ; % work item

check = check + keep * 10^(Left2Right - 1) ; % validate

end % for each local DOF

if ( flag > check )                 ; % check for likely error

fprintf ('WARNING: bc flag likely reversed at node %g. \n', N)

end % if likely user error

% end unpack_pt_flags

## References

J.L. Meek, Matrix Structural Analysis, McGraw-Hill, 1971.

## Appendix – Checking with MODEL f90

*** INPUT SOURCE RESULTANTS ***

ITEM       SUM           POSITIVE      NEGATIVE

1    0.0000E+00    0.0000E+00    0.0000E+00

2   -1.0000E+01    0.0000E+00   -1.0000E+01

*** REACTION RECOVERY ***

NODE, PARAMETER,     REACTION, EQUATION

1,  DOF_1,       4.4409E-16     1

1,  DOF_2,       5.0000E+00     2

6,  DOF_2,       5.0000E+00    12

REACTION RESULTANTS

PARAMETER,    SUM          POSITIVE     NEGATIVE

DOF_1,      4.4409E-16   4.4409E-16   0.0000E+00

DOF_2,      1.0000E+01   1.0000E+01   0.0000E+00

*** EXTREME VALUES OF THE NODAL PARAMETERS ***

PARAMETER      MAXIMUM,   NODE      MINIMUM,   NODE

DOF_1,     1.6667E-03,      2  -1.5639E-04,      5

DOF_2,     0.0000E+00,      1  -4.4013E-03,      3

***  OUTPUT OF RESULTS IN NODAL ORDER  ***

NODE,  X-Coord,    Y-Coord,    DOF_1,      DOF_2,

1  0.0000E+00  0.0000E+00  0.0000E+00  0.0000E+00

2  0.0000E+00  1.0000E+01  1.6667E-03 -9.1153E-04

3  1.0000E+01  0.0000E+00  7.5514E-04 -4.4013E-03

4  1.0000E+01  1.0000E+01  7.5514E-04 -2.8910E-03

5  2.0000E+01  1.0000E+01 -1.5639E-04 -9.1153E-04

6  2.0000E+01  0.0000E+00  1.5103E-03  0.0000E+00

** BEGIN ELEMENT APPLICATION POST PROCESSING **

E L E M E N T    S T R E S S E S

ELEMENT      MID SECTION STRESS AT:

NUMBER     RIGHT                LEFT

1       0.2265409E+01       0.2265409E+01

2       0.2265409E+01       0.2265409E+01

3      -0.2734591E+01      -0.2734591E+01

4       0.3867295E+01       0.3867295E+01

5      -0.3203772E+01      -0.3203772E+01

6       0.4530818E+01       0.4530818E+01

7      -0.3203772E+01      -0.3203772E+01

8       0.3867295E+01       0.3867295E+01

9      -0.2734591E+01      -0.2734591E+01

10      -0.2734591E+01      -0.2734591E+01

11      -0.2734591E+01      -0.2734591E+01

NORMAL END OF MODEL_F90, WITH  2  WARNINGS